Assignment.2 (Group) – Reliable Data Transfer over UDP

CP372 – Computer Network, Spring 2026

1. Overview

In this assignment, you will implement and compare two fundamental reliable data transfer protocols:

- Stop-and-Wait

- Go-Back-N (GBN)

Both protocols will be implemented on top of UDP sockets in Python.

Since UDP does not provide reliability, your implementation must add mechanisms such as:

- Sequence numbers

- Acknowledgments (ACKs)

- Timeouts

- Retransmissions

The objective is to understand how transport protocols achieve reliability despite packet loss.

2. Learning Objectives

After completing this assignment, students should be able to:

- Explain the limitations of UDP.

- Implement reliable data transfer mechanisms.

- Implement Stop-and-Wait.

- Implement Go-Back-N.

- Handle packet loss through retransmissions.

- Compare protocol performance under different network conditions.

- Analyze throughput and efficiency of pipelined transmission.

3. System Requirements

- Python 3.x

- UDP sockets (socket library)

- Standard Python libraries only

No third-party networking frameworks are permitted.

4. Project Structure & deliverables

Students should implement:

sender_stopwait.py

receiver_stopwait.py

sender_gbn.py

receiver_gbn.py

report.pdf

5. Packet Format

Design a packet structure that supports reliable data transmission. Suggested fields include:

| Field | Description |
| --- | --- |
| Sequence Number | Packet identifier |
| ACK Number | Used for acknowledgments |
| Type | DATA, ACK, and some other user defined TYPEs for file, message, control code transfer |
| Payload | Data bytes |

You may choose your own encoding format. The packet format must be elaborated in detail in the report.

Part A: Stop-and-Wait Protocol (35 Marks)

Requirements

Implement Stop-and-Wait.

Sender

The sender must:

- Read a text file,  or generate some random/constant content by the program.

- notify the server a file is to be transferred over

- Divide the file into packets.

- Send one packet at a time.

- Start a timer after transmission.

- Wait for ACK.

- Retransmit upon timeout.

- Continue until all packets are acknowledged.

- Send a packet to notify the completion of file transfer

Receiver

The receiver must:

- Receive packets.

- Check sequence number, Send ACKs for in order packets

- Check packet TYPE, create a file if the TYPE = “upload a file”, save file content if TYPE == DATA; close the file if the TYPE == “completion of file transfer”

- Ignore duplicate packets or out order packets.

Part B: Simulated Packet Loss (15 Marks)

Modify the receiver so that packets are randomly dropped.

Example:

if random.random() < LOSS_RATE:

drop packet

Test:

0%

10%

20%

30%

packet loss rates.

Demonstrate

The sender should still successfully transfer the file through retransmissions.

Part C: Go-Back-N Protocol (35 Marks)

Implement Go-Back-N using a sliding window.

Sender Requirements

Maintain:

base

nextseqnum

window_size

Behavior:

- Send multiple packets without waiting.

- Start timer for oldest unacknowledged packet.

- Process cumulative ACKs.

- Retransmit window after timeout.

Example

Window Size = 4

Send:

0 1 2 3

ACK0 ACK1 ACK2

Send:

4 5 6

Receiver Requirements

Receiver:

- Accept only the expected packet.

- Discard out-of-order packets.

- Send cumulative ACKs.

Example:

Expected = 3

Receive 4

Discard 4

Send ACK2

Packet Loss Testing

Repeat experiments using:

0%

10%

20%

30%

loss rates.

Part D: Performance Evaluation (15 Marks)

Conduct experiments comparing:

Stop-and-Wait

versus

Go-Back-N

Metrics

Measure:

1. Average Total Transfer Time (5 tests per file size per loss rate)

seconds

2. Average Throughput (5 tests per file size per loss rate)

bytes/second

3. average Number of Retransmissions (5 tests per file size per loss rate)

count

4. Protocol Efficiency

Discuss:

- Channel utilization

- Effect of packet loss

- Scalability

6. Experimental Scenarios

Use the following file sizes for testing:

10KB, 50KB, 100KB, 500KB, 1MB, 5MB, 10MB, 50MB, 100MB

Test each file size under the following loss conditions, 5 tests each:

0% loss; 10% loss; 20% loss; 30% loss

7. Report Requirements

Submit a report containing:

Introduction

Brief explanation of:

- The goal of the assignment.

- A brief introduction of the tech background including brief introductions of

  - UDP

  - general principle of reliability

  - the Stop-and-Wait protocol model

  - Go-Back-N protocol model

Design - Separate (sub)sections for each model

For each model, describe:

- Packet format

- Timer design

- ACK handling

- Window implementation

- A flow chart to show how packet loss are handled

Experimental Results

Include tables:

Table.1 Average Stop-and-Wait Transfer Time (average of at least 5 tests per file size per loss rate)

| Loss Rate | 10K | 50K | 100K | 500K | 1M | 5M | 10M | 50M | 100M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0% |  |  |  |  |  |  |  |  |  |
| 10% |  |  |  |  |  |  |  |  |  |
| 20% |  |  |  |  |  |  |  |  |  |
| 30% |  |  |  |  |  |  |  |  |  |

Table.2 Average GBN transfer Time (average of at least 5 tests per file size per loss rate)

| Loss Rate | 10K | 50K | 100K | 500K | 1M | 5M | 10M | 50M | 100M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0% |  |  |  |  |  |  |  |  |  |
| 10% |  |  |  |  |  |  |  |  |  |
| 20% |  |  |  |  |  |  |  |  |  |
| 30% |  |  |  |  |  |  |  |  |  |

Graphics

Use graphics to make visual comparison of the performance

Analysis

Discuss:

- Is GBN always faster than Stop-and-Wait for all testing cases. If yes, explain Why GBN is faster than Stop-and-Wait. If not, explain what the reason could be.

- Impact of increasing packet loss.

- Trade-offs between simplicity and efficiency.

- Situations where Stop-and-Wait may still be useful.

Conclusion

Summarize findings and suggest possible improvements to the overall throughput

8. Bonus Tasks (+10 Marks)

Choose one:

Option A (10% extra)

Replace GBN with Selective Repeat (but using only one timer).

OR

Option B (5% extra)

Add packet corruption simulation:

if random.random() < CORRUPTION_RATE:

corrupt packet

define and use checksums for error detection.

9. Marking Scheme

| Component | Marks |
| --- | --- |
| Stop-and-Wait Sender | 10 |
| Stop-and-Wait Receiver | 10 |
| Packet Loss Handling | 10 |
| Go-Back-N Sender | 15 |
| Go-Back-N Receiver | 15 |
| Experimental Evaluation | 15 |
| Report & presentation | 20 |
| Code Quality | 5 |
| Total | 100 |

Submission Instructions

containing:

sender_stopwait.py

receiver_stopwait.py

sender_gbn.py

receiver_gbn.py

report.pdf

a video presentation of the major functionalities and a demo of execution.

Readme.txt - Include instructions for running your programs and any assumptions made during implementation.

Bonus.txt – describe the extra efforts for consideration of bonus marks